# DriveMind: Proactive V2X Collective Safety Graph & Driver Intent Intelligence

A Submission for Tata Technologies

## Slide 1: Introduction (Team & Project Overview)

### The Team

- Project Developers: Advanced Agentic AI pair programming initiative.

- Project Sponsor/Focus: Tata Technologies Innovation Challenge.

### Project Overview: DriveMind

- Concept: A cooperative automotive safety platform that moves ADAS (Advanced Driver Assistance Systems) from reactive alert triggers to proactive intent prediction.

- Core Mechanisms:

  - Real-time sequence prediction of driver actions (e.g., lane-changing, braking, turning) 1.5 seconds in advance.

  - Spatial-semantic “Collective Memory” graph mapping road grids and localized weather hazards.

  - Real-time V2X warnings broadcasted to connected vehicles in the same network grid.

## Slide 2: Problem Statement

### The Challenge

- Reactive ADAS Limitations: Current collision-avoidance systems warn drivers during or after an emergency begins (e.g., lane drift or hard brakes), leaving less than 0.5s of human reaction buffer.

- Siloed Telemetry (No V2X Collaboration): Modern vehicles operate in isolated silos. When one car encounters a hazard (like curve slippage or fog), surrounding cars receive no warning, leading to pile-ups.

- Noise vs. Intent: Separating normal driver steering jitter from dangerous, deliberate maneuvers in real-time requires deep sequence analytics.

### Market Size & Future Potential

- ADAS Market Size: Valued at USD 30+ Billion globally, projected to reach USD 74 Billion by 2030 (CAGR of 12%+).

- V2X (Vehicle-to-Everything) Market: Accelerating with 5G rollout, expected to hit USD 10+ Billion by 2028.

- Potential Impact:

  - Prevents up to 45% of multi-vehicle pile-ups on curves and highways.

  - Reduces traffic delays by 15-20% via cooperative routing recommendations.

## Slide 3: Objective & Approach

### Our Main Goal

To build a high-performance, cooperative safety network that anticipates driver maneuvers with >92% accuracy and broadcasts proactive spatial warnings to connected fleets using a shared hazard memory graph.

### Methodology & Workflow

- Dense Telemetry Windowing: Capture a sliding 45-frame sequence of streaming coordinates, steering angles, speed, and braking inputs.

- Dynamic Intent Modeling: Process inputs through an 80-feature machine learning model to categorize the driver’s next action before it occurs.

- Graph Collective Memory: Write incident logs as semantic RDF triples (Sector ↔ Threat ↔ Weather) to a Neo4j database.

- Proactive V2X Warnings: Push safety alerts and step-by-step driver instructions (HUD) via low-latency WebSockets.

## Slide 4: Solution Overview (Key Features & Novelty)

graph TD
    A[Connected Vehicles] -->|Real-Time Telemetry| B[FastAPI AI Service]
    B -->|Predict Intention 92.2%| C[Node.js backend]
    A -->|Coordinate Tracking| C
    C -->|Aggregate Experience Ledger| D[(MongoDB)]
    C -->|RDF Graph Relationships| E[(Neo4j Database)]
    E -->|Cypher Similarity Engine| C
    C -->|V2X Warning WebSockets| A

### 1. Real-Time Driver Intent Engine (Novelty)

- Sliding 45-frame sequence analysis mapping rolling averages, standard deviations, and lag differentials.

- Forecasts maneuvers 1.5 seconds early, giving drivers ~21 meters of extra braking space at 50 km/h.

### 2. Cooperative Memory Graph

- Neo4j structures road sectors, weather conditions, and incident types.

- Allows vehicles to query similar hazard threats across the network using graph node similarity algorithms.

## Slide 5: Solution Overview (User Dashboard & Simulator)

### The Multi-Vehicle Cockpit Simulator

- Generates live telemetry feeds from multiple cars (leading/following vehicles).

- Playback scripts for dangerous driving scenarios:

  - Unsafe Tailgating: High speed with <5m following gap.

  - Curve Speeding: Sharp steering angles with traction loss on wet curves.

  - Fog Emergency Braking: Abrupt deceleration under zero visibility.

### Operator Control Hub

- SVG Trend Visualizer: Plots 7-day historical averages of risk parameters to identify danger patterns.

- Sector Leaderboards: Lists top danger sectors ranked by incident frequency.

- Drill-down Similarity Panel: Identifies adjacent curves or intersections showing identical threat vectors.

## Slide 6: Technical Implementation

### Backend & Database Infrastructure

- Node.js & Express.js: Central service layer orchestrating API routing and rules engines.

- FastAPI (Python 3.12): Microservice for low-latency machine learning inference (<20ms response time).

- MongoDB: Document database for raw telemetry logs and experience ledgers.

- Neo4j: Graph database for spatial safety mappings and hazard relational networks.

- Socket.io: WebSockets framework driving V2X hazard warning broadcasts.

### Machine Learning Stack

- Extra Trees Classifier (500 Estimators): Selected after comparative iterations against Random Forest and XGBoost.

- Scikit-Learn, Pandas & NumPy: For data cleansing, smoothing, feature extraction, and model training.

### Frontend Interface

- React & Vite: Core frontend UI workspace.

- React-Leaflet: Visual mapping of real-time vehicle GPS tracks and hazardous segment grids.

- Tailwind CSS: Premium styling for driver cockpit simulation HUD.

## Slide 7: Challenges Faced & Mitigation

### 1. AI Model Overfitting & Feature Noise

- Challenge: Synthetic datasets led to trivial 98%+ training scores but failed to adapt to real driving trajectories.

- Mitigation: Trained the model on the real-world I See You trajectory dataset. Re-engineered the features using a rolling 15-frame Gaussian smoothing filter to eliminate GPS noise, resulting in a robust, generalizable 92.2% test accuracy.

### 2. Docker Container Memory Crashes (OOM)

- Challenge: Large ML models (1,500 decision trees) consumed over 1.5GB of RAM, exceeding Docker Desktop limits on development systems and triggering Exit Code 137.

- Mitigation: Reduced estimators to a lightweight 500-tree model, keeping performance at 92.2% while reducing RAM usage by 3x. Moved FastAPI hosting natively to the macOS host virtual environment, routing container traffic seamlessly via host.docker.internal.

### 3. Database Sync Lag

- Challenge: Real-time database calls slowed down WebSocket response rates during rapid telemetry updates.

- Mitigation: Optimized Mongo aggregation pipelines and structured indices, keeping total payload processing under 25ms.

## Slide 8: Future Enhancements

### 1. Edge Micro-Classifier Deployment

- Compile the Extra Trees classifier into ONNX or TensorRT formats. This allows models to run directly on vehicle ECUs (Edge AI) without needing constant internet connection to the cloud.

### 2. Deep Learning Sequence Models (LSTM / Transformers)

- Transition from tabular ensemble trees to sequence models (Long Short-Term Memory networks or Transformer encoders) to process longer, continuous trajectory segments and forecast multi-step driving behaviors.

### 3. Autonomous ADAS Actuator Loop

- Connect the V2X warning pipeline directly to autonomous braking and steering controls (Hardware-in-the-Loop simulation), enabling active steering torque assistance or automated emergency deceleration based on predicted intent.

## Slide 9: Project Plan (Prototype to Production PoC)

### Phase 1: Research & Setup (Completed)

- Real-world dataset processing, target variable definitions, and feature engineering.

- Docker Compose multi-container stack configuration.

### Phase 2: Core Algorithm Iteration (Completed)

- Intent model training, parameter tuning, and validation testing.

- Relational schema creation on MongoDB & Neo4j databases.

### Phase 3: Integration & Dashboard UI (Completed)

- WebSocket connection pipelines and Leaflet tracking maps.

- Multi-vehicle playback simulator.

### Phase 4: Field Test PoC (Next Steps)

- Deploy edge telemetry scripts on test vehicles.

- Validate V2V sharing latency under real 5G and LTE cellular conditions.

- Complete hardware validation on an embedded computer board (e.g., NVIDIA Jetson).